TEMPERATURE BOT

Site Manual

The Hubitat installation, hub by hub and device by device

Surveyed 20 August 2026

Hubs 10.2.3.51 - 10.2.3.54, Somerville

A snapshot. Regenerate from the hubs before relying on any number.

Contents

The Four Hubs

What Temperature Bot Can Actually See

The Hub Mesh Map

Other Things That Command This Hardware

Hub 10.2.3.51 — Somerville Area 51

Hub 10.2.3.52 — Somerville Broadway

Hub 10.2.3.53 — Somerville Greenhouse

Hub 10.2.3.54 — CALA Garage

Health Notes

Re-running This Survey

A census of the Hubitat installation: which hubs exist, what is paired to each, which apps run on them, and exactly which devices Temperature Bot can see.

doc/hardware-landscape.md is the companion to this file and should be read first. It explains the concepts — what a Hubitat hub is, what Z-Wave and Zigbee are, the difference between pairing, Hub Mesh, and Maker API exposure, and why a device on a wall dashboard may be invisible to us. This file explains what is actually installed, and deliberately does not repeat the concepts.

There is a Word version of this file, doc/site-manual.docx, for readers who want something printable. It is generated, not maintained. After editing this file run make site-manual-docx to rebuild it; never edit the .docx directly, because the next rebuild silently discards those edits.

This is a snapshot, taken 2026-08-20. Devices get added, renamed, and re-meshed without anyone updating a document. Treat the tables here as a starting map, not as truth; "Re-running This Survey" at the end gives the read-only commands that regenerate every number on this page in about a minute.

The Four Hubs

There are four Hubitat hubs on the office LAN, not two. All four are the same model on the same firmware, all four answer HTTP on the LAN with no authentication, and all four report their radios healthy.

| Host | Hub name | Devices | Top-level apps | What it is for |
| --- | --- | --- | --- | --- |
| 10.2.3.51 | Somerville Area 51 | 120 (83 native, 37 meshed in) | 12 | The hub Temperature Bot talks to. Area 51, Hickory, Dungeon, Huddle and Meeting rooms, plus everything meshed in from .52 and .53. |
| 10.2.3.52 | Somerville Broadway | 40 | 8 | Broadway, Lobby, Kitchen, Outside. Offers 33 devices to the mesh; 29 are linked onto .51. |
| 10.2.3.53 | Somerville Greenhouse | 11 | 5 | The Greenhouse and the Lobby color/intensity dimmers. Offers 8 devices; all 8 are linked onto .51. |
| 10.2.3.54 | CALA Garage | 15 | 4 | Garage parking-spot sensors and heaters. Shares nothing, runs no Maker API, and is invisible to every other hub and to us. |

Common to all four, as of the survey:

– Model C-8 Pro, platform 2.5.0.159, with 2.5.1.152 available. Nobody should apply that update casually: .51 is what feeds every reading we store.

– Temperature scale C, matching our temp10x storage.

– No zwaveOffline, zigbeeOffline, load, or memory alerts on any hub.

The app column counts top-level app instances only, the same way the device column would mislead if it stopped at the top level. Rule Machine is one instance holding ten rules; a Dashboards instance holds five dashboards. Counting every nested child the totals are 44, 32, 15, and 5.

One trap in the JSON: baseModel.zigbeeStatus and baseModel.zwaveStatus read "false" on all four hubs and this does not mean the radios are off — .51 has 39 Z-Wave devices, most of which had reported within the hour the survey ran. Those fields describe a UI state. For radio health read alerts.zwaveOffline and alerts.zigbeeOffline.

What Temperature Bot Can Actually See

Everything we read and every command we send goes through one Maker API app: app 520, "Maker API for Temperature Bot", on 10.2.3.51. It exposes 31 devices. That is the entire hardware surface of this application. The four hubs hold 186 device rows between them, 37 of which are mesh duplicates, so 149 distinct devices exist in the building and 118 of them may as well not, as far as our code is concerned.

| .51 id | Label Maker API reports | Device type | Where it comes from |
| --- | --- | --- | --- |
| 98 | Dungeon Cage | AeotecMultiSensor6 | native to .51 |
| 129 | A51-1 | AeotecMultiSensor6 | native to .51 |
| 130 | A51-2 | AeotecMultiSensor6 | native to .51 |
| 134 | A51-3 | AeotecMultiSensor6 | native to .51 |
| 135 | A51-4 | AeotecMultiSensor6 | native to .51 |
| 260 | Broadway Pendant Lights | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 291 |
| 354 | Sidewalk Washer North | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 294 |
| 355 | Sidewalk Washer South | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 295 |
| 356 | Whiteboard Washer | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 293 |
| 359 | Data Closet Fan | GE Smart Fan Control | meshed from 10.2.3.52 id 137 |
| 360 | Garage Washer North | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 136 |
| 361 | Garage Washer South | GE Enbrighten Z-Wave Smart Switch | meshed from 10.2.3.52 id 297 |
| 454 | Green Wall - Inner | GE Enbrighten Z-Wave Smart Switch | native to .51 |
| 522 | Lobby Sensor | AeotecMultiSensor6 | meshed from 10.2.3.52 id 38 |
| 523 | Greenhouse East | AeotecMultiSensor6 | meshed from 10.2.3.53 id 35 |
| 524 | Greenhouse West | AeotecMultiSensor6 | meshed from 10.2.3.53 id 34 |
| 525 | Broadway Center | AeotecMultiSensor6 | meshed from 10.2.3.52 id 37 |
| 526 | Broadway North | AeotecMultiSensor6 | meshed from 10.2.3.52 id 36 |
| 527 | Broadway South | AeotecMultiSensor6 | meshed from 10.2.3.52 id 151 |
| 528 | Data Closet Sensor | AeotecMultiSensor6 | meshed from 10.2.3.52 id 138 |
| 532 | A51 Hallway Sensor | AeotecMultiSensor6 | native to .51 |
| 550 | Green Wall - Outer | Minoston MP31ZP Smart Plug | native to .51 |
| 566 | Hickory Lights Remote | Aeon WallMote | native to .51 |
| 581 | Hickory Dimmable Lights | hueBridgeGroup | native to .51 |
| 610 | Hickory TV Up/Down Relay | Zooz ZEN17 Universal Relay Advanced | native to .51 |
| 611 | TV Up | Generic Component Switch | child of 610 (Hickory TV Up/Down Relay) |
| 612 | TV Down | Generic Component Switch | child of 610 (Hickory TV Up/Down Relay) |
| 616 | Broadway Spot Lights | Minoston Mini Power Meter Plug | meshed from 10.2.3.52 id 393 |
| 617 | Hickory Sensor | AeotecMultiSensor6 | native to .51 |
| 618 | Broadway TV Cart Left on Somerville Broadway | Generic Z-Wave Plus Outlet | meshed from 10.2.3.52 id 395 |
| 619 | Broadway TV Cart Right on Somerville Broadway | Generic Z-Wave Plus Outlet | meshed from 10.2.3.52 id 396 |

Three things in that table are worth stopping on.

611 and 612 are child devices. The Zooz ZEN17 relay (610) creates two component switches under itself, and Hubitat's device list nests them inside their parent. A tool that reads only the top level of /hub2/devicesList counts 115 devices on .51 and never sees these two — which is why this file says 120 and older notes say 115. Both numbers came from the same hub on the same day.

618 and 619 carry their hub name in their label. Maker API reports them as "Broadway TV Cart Left on Somerville Broadway". That suffix is part of the label as meshed, not something we add. app/room_config.py overrides the display name, so the UI says "TV Cart Left"; the raw label still reads the long way in any Maker API output.

More than half of what we read is not paired to .51 at all. Seventeen of the 31 arrive by Hub Mesh from .52 or .53. That is the subject of the next section, and it is the single most common source of wrong device ids here.

The Hub Mesh Map

Hub Mesh shares an already-paired device from one hub onto another. The receiving hub treats it as a first-class device — but assigns it a new id. The same physical sensor is 37 on .52 and 525 on .51. An id copied from another hub's dashboard or URL is at best dead on .51 and at worst names an unrelated device; this has already caused a real bug, when three Broadway control ids taken from .52 turned out on .51 to be Kitchen Counter Lights, Cedar Lights, and Willow Lights.

This table is the translation. Every device on .51 that came from somewhere else, with the id and name it has at its source.

| .51 id | Name on .51 | Source hub | Source id | Name on the source hub | App 520 |
| --- | --- | --- | --- | --- | --- |
| 257 | Bamboo Lights | 10.2.3.52 | 3 | Bamboo Lights | — |
| 258 | Kitchen Main Lights | 10.2.3.52 | 290 | Kitchen Main Lights | — |
| 259 | Lobby Main Lights | 10.2.3.52 | 289 | Lobby Main Lights | — |
| 260 | Broadway Pendant Lights | 10.2.3.52 | 291 | Broadway Pendant Lights | yes |
| 289 | Lobby Long Grazers | 10.2.3.52 | 66 | Lobby Long Grazers | — |
| 290 | Lobby Short Grazers | 10.2.3.52 | 65 | Lobby Short Grazers | — |
| 291 | Kitchen Counter Lights | 10.2.3.52 | 40 | Kitchen Counter Lights | — |
| 292 | Main Entrance Lights | 10.2.3.52 | 292 | Main Entrance Lights | — |
| 293 | Cedar Lights | 10.2.3.52 | 97 | Cedar Lights | — |
| 294 | Willow Lights | 10.2.3.52 | 98 | Willow Lights | — |
| 353 | Column & Sidewalk Lights | 10.2.3.52 | 296 | Column & Sidewalk Lights | — |
| 354 | Sidewalk Washer North | 10.2.3.52 | 294 | Sidewalk Washer North | yes |
| 355 | Sidewalk Washer South | 10.2.3.52 | 295 | Sidewalk Washer South | yes |
| 356 | Whiteboard Washer | 10.2.3.52 | 293 | Whiteboard Washer | yes |
| 357 | Broadway Lights | 10.2.3.52 | 132 | Broadway Lights | — |
| 358 | Lobby Lights | 10.2.3.52 | 133 | Lobby Lights | — |
| 359 | Data Closet Fan | 10.2.3.52 | 137 | Data Closet Fan | yes |
| 360 | Garage Washer North | 10.2.3.52 | 136 | Garage Washer North | yes |
| 361 | Garage Washer South | 10.2.3.52 | 297 | Garage Washer South | yes |
| 368 | Garden Lights | 10.2.3.52 | 141 | Garden Lights | — |
| 380 | Greenwall White Lights | 10.2.3.53 | 3 | Greenwall White Lights | — |
| 381 | Greenwall Yellow Lights | 10.2.3.53 | 4 | Greenwall Yellow Lights | — |
| 382 | Greenhouse Main Lights | 10.2.3.53 | 97 | Greenhouse Main Lights | — |
| 417 | Greenhouse Intensity | 10.2.3.53 | 36 | Greenhouse Intensity | — |
| 418 | Lobby Color | 10.2.3.53 | 65 | Lobby Color | — |
| 419 | Lobby Intensity | 10.2.3.53 | 66 | Lobby Intensity | — |
| 519 | Broadway Power Monitor | 10.2.3.52 | 387 | Broadway Power Monitor | — |
| 522 | Lobby Sensor | 10.2.3.52 | 38 | Lobby Sensor | yes |
| 523 | Greenhouse East | 10.2.3.53 | 35 | Greenhouse Sensor East | yes |
| 524 | Greenhouse West | 10.2.3.53 | 34 | Greenhouse Sensor West | yes |
| 525 | Broadway Center | 10.2.3.52 | 37 | Broadway Sensor Center | yes |
| 526 | Broadway North | 10.2.3.52 | 36 | Broadway Sensor North | yes |
| 527 | Broadway South | 10.2.3.52 | 151 | Broadway Sensor South | yes |
| 528 | Data Closet Sensor | 10.2.3.52 | 138 | Data Closet Sensor | yes |
| 616 | Broadway Spot Lights | 10.2.3.52 | 393 | Broadway Spot Lights | yes |
| 618 | Broadway TV Cart Left on Somerville Broadway | 10.2.3.52 | 395 | Broadway TV Cart Left | yes |
| 619 | Broadway TV Cart Right on Somerville Broadway | 10.2.3.52 | 396 | Broadway TV Cart Right | yes |

Twenty-nine of these come from .52 and eight from .53. .54 contributes nothing. Note that .52 offers thirty-three — four window shades are shared but never linked, so they are absent from this table; see the .52 section.

Devices `.51` shares outward

.51 also offers twenty of its own devices to the mesh. Nothing currently consumes them — .52, .53, and .54 have no linked devices at all, and only another Hubitat hub can link a meshed device, so this is not how the Home Assistant integration below reaches anything. Most likely they are leftovers from an earlier arrangement. Either way, sharing a device out has no effect on what Temperature Bot sees; only app 520 decides that.

| .51 id | Name | Hub room |
| --- | --- | --- |
| 33 | Shower Room Lights | Shower Room |
| 66 | Dungeon Ceiling | Dungeon |
| 98 | Dungeon Cage | Dungeon |
| 100 | Restroom Lights | Restrooms |
| 101 | Maple Lights | Meeting Rooms |
| 129 | A51-1 | Area 51 |
| 130 | A51-2 | Area 51 |
| 131 | Banyan Lights | Meeting Rooms |
| 132 | Eucalyptus Lights | Meeting Rooms |
| 133 | Sequoia Lights | Meeting Rooms |
| 134 | A51-3 | Area 51 |
| 135 | A51-4 | Area 51 |
| 163 | Huddle 1 Lights | Huddle Rooms |
| 164 | Huddle 2 Lights | Huddle Rooms |
| 193 | Huddle 3 Lights | Huddle Rooms |
| 194 | Huddle 4 Lights | Huddle Rooms |
| 195 | Huddle 5 Lights | Huddle Rooms |
| 225 | Huddle 6 Lights | Huddle Rooms |
| 532 | A51 Hallway Sensor | Area 51 |
| 617 | Hickory Sensor | Hickory |

Other Things That Command This Hardware

We are not the only consumer, and this matters when a switch changes state and nothing in our logs explains why.

– Maker API app 493, "Maker API for Home Assistant", runs on .51 alongside ours. It is a second programmable allowlist over the same hub, with its own token and its own device selection, and whatever holds that token can read and command those devices exactly as we can. Its token is not in temperature-bot-config.yaml, so which devices it exposes is unverified — this survey establishes that the app exists, not what it reaches.

– Rule Machine, Motion Lighting, and Button Controller apps run automations entirely inside the hubs. .51 alone has ten Rule Machine rules and three motion-lighting apps. A light that turns itself off at sunrise is doing so because of a rule on the hub, not because of app/rules_engine.py.

– Hubitat's own dashboards are display surfaces for humans, on wall tablets and phones. Eight are installed across three hubs. A device on a dashboard is not thereby visible to us.

– The Hue Bridge (BC6E12, app 521 on .51) contributes 26 bulbs and 5 groups in Hickory. Only the 581 group is exposed to us.

– Mobile presence apps. Several phones are registered as devices across the hubs. None are exposed to app 520.

Hub 10.2.3.51 — Somerville Area 51

The only hub in our configuration, and the only one our code can reach: temperature-bot-config.yaml holds exactly one hubitat.host and one hubitat.appId, so supporting a second hub would mean changing the configuration model, not just adding a token.

83 devices are native to this hub; the other 37 are meshed in and listed in the map above. In the table below (shared) marks a device shared outward to the mesh, and (520) marks one exposed through app 520.

| id | Hub room | Name | Protocol | Device type | Last seen |
| --- | --- | --- | --- | --- | --- |
| 129 | Area 51 | A51-1 (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 130 | Area 51 | A51-2 (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 134 | Area 51 | A51-3 (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 135 | Area 51 | A51-4 (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 321 | Area 51 | Area 51 East Column Outlet | Z-Wave | Generic Z-Wave Plus Outlet | 2025-06-25 |
| 364 | Area 51 | Garden Door Relay | Z-Wave | Zooz MultiRelay | 2026-08-17 |
| 365 | Area 51 | Garden Overhead Door | child | Generic Component Switch | 2026-08-17 |
| 366 | Area 51 | Garden Door Spare 2 | child | Generic Component Switch | 2025-07-18 |
| 367 | Area 51 | Garden Door Spare 3 | child | Generic Component Switch | 2025-07-18 |
| 421 | Area 51 | A51 Dimmer #1 | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 422 | Area 51 | A51 Dimmer #2 | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 450 | Area 51 | A51 Dimmer #3 | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 451 | Area 51 | A51 Dimmer #4 | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 453 | Area 51 | Area 51 Pendant Lights | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 481 | Area 51 | Area 51 Main Lights | software | Group Dimmer | 2026-08-20 |
| 517 | Area 51 | NO PULP | Z-Wave | Generic Z-Wave Plus Outlet | 2026-08-20 |
| 520 | Area 51 | Area 51 Meeting Rooms | software | Group Switch | 2026-06-10 |
| 532 | Area 51 | A51 Hallway Sensor (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 534 | Area 51 | Water Sensor 2 | Z-Wave | Zooz ZSE42 Water Leak XS Sensor | 2026-08-20 |
| 571 | Area 51 | A51 Dimmer #5 | Z-Wave | Zooz Zen54 0-10V Dimmer | 2026-08-20 |
| 570 | Broadway | Broadway Washers | software | Group Switch | 2026-08-20 |
| 66 | Dungeon | Dungeon Ceiling (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 98 | Dungeon | Dungeon Cage (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 384 | Dungeon | Dungeon Meter | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 449 | Dungeon | Dungeon Lights | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 518 | Dungeon | Dungeon Power Monitor | Z-Wave | Minoston Mini Power Meter Plug | 2026-08-20 |
| 562 | Dungeon | Dungeon Exhaust Fan | Z-Wave | Generic Z-Wave Plus Scene Switch | 2026-08-20 |
| 454 | Hickory | Green Wall - Inner (520) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 550 | Hickory | Green Wall - Outer (520) | Z-Wave | Minoston MP31ZP Smart Plug | 2026-08-20 |
| 556 | Hickory | Hickory Lights | software | Group Switch | 2026-08-20 |
| 566 | Hickory | Hickory Lights Remote (520) | Z-Wave | Aeon WallMote | 2026-08-20 |
| 572 | Hickory | hueBridge BC6E12 (Hue Bridge) | software | hueBridge | 2026-08-20 |
| 581 | Hickory | Hickory Dimmable Lights (520) | software | hueBridgeGroup | 2026-08-20 |
| 605 | Hickory | Green Wall | software | Group Switch | 2026-08-20 |
| 606 | Hickory | Hickory Lights - Ceiling Front | software | hueBridgeGroup | 2026-08-20 |
| 607 | Hickory | Hickory Lights - Ceiling Rear | software | hueBridgeGroup | 2026-08-20 |
| 608 | Hickory | Hickory Lights - Left Wall | software | hueBridgeGroup | 2026-08-20 |
| 609 | Hickory | Hickory Lights - Right Wall | software | hueBridgeGroup | 2026-08-20 |
| 610 | Hickory | Hickory TV Up/Down Relay (520) | Z-Wave | Zooz ZEN17 Universal Relay Advanced | 2026-08-08 |
| 611 | Hickory | TV Up (520) | child | Generic Component Switch | 2026-08-08 |
| 612 | Hickory | TV Down (520) | child | Generic Component Switch | 2026-08-08 |
| 617 | Hickory | Hickory Sensor (shared) (520) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 573 | Hickory Hue Bulbs | MS-A1 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 574 | Hickory Hue Bulbs | MS-A2 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 575 | Hickory Hue Bulbs | MS-A3 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 576 | Hickory Hue Bulbs | MS-A4 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 577 | Hickory Hue Bulbs | MS-A5 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 578 | Hickory Hue Bulbs | MS-A6 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 579 | Hickory Hue Bulbs | MS-A7 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 580 | Hickory Hue Bulbs | MS-A8 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 583 | Hickory Hue Bulbs | MS-B1 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 584 | Hickory Hue Bulbs | MS-B2 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 585 | Hickory Hue Bulbs | MS-B3 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 586 | Hickory Hue Bulbs | MS-B4 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 587 | Hickory Hue Bulbs | MS-B5 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 588 | Hickory Hue Bulbs | MS-B6 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 589 | Hickory Hue Bulbs | MS-B7 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 590 | Hickory Hue Bulbs | MS-B8 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 593 | Hickory Hue Bulbs | MS-C1 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 594 | Hickory Hue Bulbs | MS-C2 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 595 | Hickory Hue Bulbs | MS-C3 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 596 | Hickory Hue Bulbs | MS-C4 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 597 | Hickory Hue Bulbs | MS-C5 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 598 | Hickory Hue Bulbs | MS-D1 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 599 | Hickory Hue Bulbs | MS-D2 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 600 | Hickory Hue Bulbs | MS-D3 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 601 | Hickory Hue Bulbs | MS-D4 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 602 | Hickory Hue Bulbs | MS-D5 | software | hueBridgeBulbRGBW | 2026-08-20 |
| 163 | Huddle Rooms | Huddle 1 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 164 | Huddle Rooms | Huddle 2 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 193 | Huddle Rooms | Huddle 3 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 194 | Huddle Rooms | Huddle 4 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 195 | Huddle Rooms | Huddle 5 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 225 | Huddle Rooms | Huddle 6 Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 420 | Huddle Rooms | Huddle 5 TV Power | Z-Wave | Generic Z-Wave Plus Outlet | 2026-08-11 |
| 101 | Meeting Rooms | Maple Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 131 | Meeting Rooms | Banyan Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 132 | Meeting Rooms | Eucalyptus Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 133 | Meeting Rooms | Sequoia Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 100 | Restrooms | Restroom Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 33 | Shower Room | Shower Room Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 539 | — | Carl's Nord N30 5G | software | Mobile App Device | 2026-04-09 |
| 615 | — | Carl's Pixel 10 Pro XL | software | Mobile App Device | 2026-04-09 |

Apps on `.51`

– 513 Basic Button Controllers

· 514 Basic Button Controller: Studio Lights Remote (7 rules)

– 510 Easy Dashboards

– 353 Groups and Scenes

· 354 Area 51 Main Lights

· 485 Area 51 Meeting Rooms

· 519 Broadway Washers

· 512 Hickory Lights

· 527 Studio Lights - Green Wall

– 65 Hubitat Z-Wave Mesh Details — community app

– 37 Hubitat® Dashboards

· 489 Area 51 Lights

· 538 Hickory

· 490 Somerville Office Doors + Fans

· 130 Somerville Office Lights

· 450 Somerville Office Sensors

– 521 Hue Bridge Integration (BC6E12 - Hue Bridge)

– 493 Maker API for Home Assistant

– 520 Maker API for Temperature Bot

– 299 Motion and Mode Lighting Apps

· 321 Area 51 Motion

· 300 Dungeon Motion

· 541 Hickory Motion

– 258 Notifications

· 486 Water Notifier

– 417 Rebooter — community app

– 226 Rule Machine

· 228 Daily indoor lights off, outdoor lights on

· 488 Daily Sunrise

· 492 Dim Greenhouse

· 386 Dungeon Exhaust Fan Off

· 385 Dungeon Exhaust Fan On

· 535 Hickory TV Down

· 537 Hickory TV Up

· 540 Movement anywhere to air1

· 484 Power Restored

· 227 Weekday Indoor Lights On

Hub 10.2.3.52 — Somerville Broadway

Broadway, Lobby, Kitchen, and the outside lighting. This hub runs no Maker API app, so it has no direct interface to us at all — everything we get from Broadway arrives through .51: the Broadway sensors, the Data Closet fan and sensor, the washers, and the TV carts.

Thirty-three of its forty devices are offered to the mesh, but only 29 are actually linked onto .51. Offering and linking are two steps, and the four that stop at the first are 353, 354, 355, and 356 — the window shades. They are shared, nothing consumes them, and they do not appear anywhere in the mesh map above.

Those four shades are also the only Zigbee devices in the entire four-hub estate. Everything else on every hub is Z-Wave or software.

(shared) marks a device offered to the mesh. It does not mean anything linked it.

| id | Hub room | Name | Protocol | Device type | Last seen |
| --- | --- | --- | --- | --- | --- |
| 3 | Bamboo | Bamboo Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 36 | Broadway | Broadway Sensor North (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 37 | Broadway | Broadway Sensor Center (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 97 | Broadway | Cedar Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-20 |
| 98 | Broadway | Willow Lights (shared) | Z-Wave | GE Z-Wave Plus Motion Switch | 2026-08-19 |
| 132 | Broadway | Broadway Lights (shared) | software | Group Switch | 2026-08-20 |
| 136 | Broadway | Garage Washer North (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 137 | Broadway | Data Closet Fan (shared) | Z-Wave | GE Smart Fan Control | 2026-08-20 |
| 138 | Broadway | Data Closet Sensor (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 151 | Broadway | Broadway Sensor South (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 291 | Broadway | Broadway Pendant Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 293 | Broadway | Whiteboard Washer (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 294 | Broadway | Sidewalk Washer North (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 295 | Broadway | Sidewalk Washer South (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 297 | Broadway | Garage Washer South (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 353 | Broadway | Window Shade 2 (shared) | Zigbee | Generic Zigbee Shade | 2026-04-27 |
| 354 | Broadway | Window Shade 1 (shared) | Zigbee | Generic Zigbee Shade | 2026-04-27 |
| 355 | Broadway | Window Shade 3 (shared) | Zigbee | Generic Zigbee Shade | 2026-08-20 |
| 356 | Broadway | Window Shade 4 (shared) | Zigbee | Generic Zigbee Shade | 2026-08-20 |
| 385 | Broadway | Broadway Wall Controller | Z-Wave | Aeon WallMote | 2026-08-20 |
| 387 | Broadway | Broadway Power Monitor (shared) | Z-Wave | Minoston Mini Power Meter Plug | 2026-08-20 |
| 393 | Broadway | Broadway Spot Lights (shared) | Z-Wave | Minoston Mini Power Meter Plug | 2026-08-20 |
| 395 | Broadway | Broadway TV Cart Left (shared) | Z-Wave | Generic Z-Wave Plus Outlet | 2026-08-20 |
| 396 | Broadway | Broadway TV Cart Right (shared) | Z-Wave | Generic Z-Wave Plus Outlet | 2026-08-17 |
| 40 | Kitchen | Kitchen Counter Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 225 | Kitchen | Jura Coffee Maker | Z-Wave | Enerwave Metering Switch ZW15RM-Plus | 2026-08-20 |
| 290 | Kitchen | Kitchen Main Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 38 | Lobby | Lobby Sensor (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 65 | Lobby | Lobby Short Grazers (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 66 | Lobby | Lobby Long Grazers (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 133 | Lobby | Lobby Lights (shared) | software | Group Switch | 2026-08-20 |
| 289 | Lobby | Lobby Main Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 386 | Lobby | Lobby Wall Controller | Z-Wave | Aeon WallMote | 2026-08-20 |
| 134 | Outside | Outside Lights | software | Group Switch | 2026-08-20 |
| 141 | Outside | Garden Lights (shared) | Z-Wave | Generic Z-Wave Plus Outlet | 2026-08-20 |
| 257 | Outside | Column Lights Color | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-04-22 |
| 292 | Outside | Main Entrance Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 296 | Outside | Column & Sidewalk Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 392 | — | Carl's Nord N30 5G | software | Mobile App Device | 2026-04-09 |
| 394 | — | Pixel 10 Pro XL | software | Mobile App Device | 2026-06-21 |

Apps on `.52`

– 417 Button Controllers

· 418 Broadway (4 rules)

· 423 Lobby (4 rules)

– 97 Groups and Scenes

· 161 Broadway Lights

· 162 Lobby Lights

· 163 Outside Lights

– 36 Hubitat Z-Wave Mesh Details — community app

– 437 Hubitat® Dashboards

· 449 Broadway Controls

– 225 Motion and Mode Lighting Apps

· 257 Broadway Motion

– 131 Notifications

· 132 Nightime Lighting Check

· 430 Power Notifier

– 353 Rebooter — community app

– 100 Rule Machine

· 432 Daily Modes

· 102 Daily Sunrise

· 103 Daily Sunset

· 448 Data Closet Fan Off

· 447 Data Closet Fan On

· 169 Lobby Motion Daytime

· 165 Lobby Motion Evening

Hub 10.2.3.53 — Somerville Greenhouse

The smallest of the office hubs: the Greenhouse itself, plus the Lobby color and intensity dimmers. Eight of its eleven devices are shared up to .51; two of those, the Greenhouse East and West sensors, are exposed to us. Like .52 it runs no Maker API app.

| id | Hub room | Name | Protocol | Device type | Last seen |
| --- | --- | --- | --- | --- | --- |
| 3 | Greenhouse | Greenwall White Lights (shared) | Z-Wave | Generic Z-Wave Plus Scene Switch | 2026-08-20 |
| 4 | Greenhouse | Greenwall Yellow Lights (shared) | Z-Wave | Generic Z-Wave Plus Scene Switch | 2026-08-20 |
| 34 | Greenhouse | Greenhouse Sensor West (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 35 | Greenhouse | Greenhouse Sensor East (shared) | Z-Wave | AeotecMultiSensor6 | 2026-08-20 |
| 36 | Greenhouse | Greenhouse Intensity (shared) | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 97 | Greenhouse | Greenhouse Main Lights (shared) | Z-Wave | GE Enbrighten Z-Wave Smart Switch | 2026-08-20 |
| 129 | Greenhouse | Greenhouse | Z-Wave | Aeon WallMote | 2026-08-06 |
| 65 | Lobby | Lobby Color (shared) | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 66 | Lobby | Lobby Intensity (shared) | Z-Wave | Qubino Flush 0-10V Dimmer | 2026-08-20 |
| 131 | — | Carl's Nord N30 5G | software | Mobile App Device | never |
| 132 | — | Leo’s iPhone 15 Pro | software | Mobile App Device | never |

Apps on `.53`

– 101 Button Controllers

· 102 Button Controller-5.1: Greenhouse (4 rules)

– 114 Hubitat® Dashboards

– 38 Motion and Mode Lighting Apps

· 39 Greenhouse Motion

– 65 Rebooter — community app

– 3 Rule Machine

· 34 Greenwall Both

· 33 Greenwall Cool

· 35 Greenwall Sleep

· 36 Greenwall Warm

Hub 10.2.3.54 — CALA Garage

A garage installation: parking-spot temperature sensors and spot heaters, plus sensors at the vehicle and pedestrian entrances. Every device is on this hub's own Z-Wave radio, in a single Hubitat room called "0F Garage".

This hub is completely isolated. It shares nothing to the mesh, no other hub has a linked device from it, and it runs no Maker API app. There is no path by which Temperature Bot could read a single value from it, and none by which it could read anything from us. Its Z-Wave Poller app is disabled.

It is also the least healthy hub in the building: ten of its fifteen devices have not reported activity in more than a month, and Spot 16 Sensor last spoke in June 2025. Whether that is a fault or simply seasonal — spot heaters have no reason to report in August — is not something this survey can tell you.

| id | Hub room | Name | Protocol | Device type | Last seen |
| --- | --- | --- | --- | --- | --- |
| 3 | 0F Garage | Spot 10 Sensor | Z-Wave | Aeotec MultiSensor 7 | 2026-05-15 |
| 4 | 0F Garage | Spot 27-28 Sensor | Z-Wave | Aeotec MultiSensor 7 | 2026-08-20 |
| 5 | 0F Garage | Spot 30 Sensor | Z-Wave | Aeon Multisensor 6 | 2026-08-20 |
| 6 | 0F Garage | Spot 22 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-08-09 |
| 7 | 0F Garage | Spot 27 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-07-07 |
| 38 | 0F Garage | Garage Entrance Sensor | Z-Wave | Aeon Multisensor 6 | 2026-08-20 |
| 39 | 0F Garage | Spot 16 Sensor | Z-Wave | Aeotec MultiSensor 7 | 2025-06-05 |
| 40 | 0F Garage | Sensor A | Z-Wave | Aeotec MultiSensor 7 | 2026-08-20 |
| 41 | 0F Garage | Spot 10 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 42 | 0F Garage | Spot 33 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 43 | 0F Garage | Spot 16 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 44 | 0F Garage | Spot 31 Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 45 | 0F Garage | Vehicle Entrance Heater Out | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 46 | 0F Garage | Package Room Heater | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |
| 47 | 0F Garage | Vehicle Entrance Heater In | Z-Wave | Generic Z-Wave Smart Switch | 2026-05-07 |

Apps on `.54`

– 1 Basic Rules

– 2 Hubitat® Dashboards

· 27 Garage

– 28 Visual Rules Builder

– 6 Z-Wave Poller — disabled (its name carries a "Polling" status badge)

Health Notes

Nothing here is an alarm. These are the things a reader of the tables above would reasonably want flagged.

A platform update is pending on all four hubs — 2.5.0.159 installed, 2.5.1.152 available. Applying it to .51 interrupts collection.

Devices that have gone quiet. Long silence is not automatically a fault: a switch that nobody touches reports nothing, and a spot heater in August has nothing to say. These are simply the ones a health check would surface.

| Hub | Device | Last activity |
| --- | --- | --- |
| .54 | Spot 16 Sensor | 2025-06-05 |
| .51 | Area 51 East Column Outlet | 2025-06-25 |
| .51 | Garden Door Spare 2, Garden Door Spare 3 | 2025-07-18 |
| .51 | Carl's Nord N30 5G, Carl's Pixel 10 Pro XL | 2026-04-09 |
| .51 | Area 51 Meeting Rooms (a group, not hardware) | 2026-06-10 |
| .52 | Column Lights Color | 2026-04-22 |
| .52 | Window Shade 1, Window Shade 2 | 2026-04-27 |
| .54 | Spot 10 Sensor | 2026-05-15 |
| .54 | Eight spot and entrance heaters | 2026-05-07 to 2026-07-07 |
| .53 | Two registered phones | never |

That is the complete set for .51, the hub we depend on. None of those six is exposed through app 520, so no reading we store is affected by any of them. For .52, .53, and .54 the table is a selection; each hub's inventory above carries a last-seen date for every device.

Database sizes are 13 MB on .51, 5 MB on .52, 4 MB on .54, 1 MB on .53, with no hubLargeishDatabase alert anywhere.

Re-running This Survey

Every table in this file comes from four read-only HTTP endpoints. The hubs require no authentication on the LAN, except for Maker API, which takes the token from temperature-bot-config.yaml under secrets.hubitat.access_token.

# Per hub: identity, model, firmware, alerts

curl -s http://10.2.3.51/hub2/hubData | python3 -m json.tool

# Per hub: every device, meshed ones included, with child devices nested

curl -s http://10.2.3.51/hub2/devicesList | python3 -m json.tool

# Per hub: installed apps, nested parent -> child -> rule

curl -s http://10.2.3.51/hub2/appsList | python3 -m json.tool

# What app 520 exposes — the only list that describes our reach

curl -s "http://10.2.3.51/apps/api/520/devices/all?access_token=$TOKEN" \

| python3 -c 'import json,sys; [print(d["id"], d["label"]) for d in json.load(sys.stdin)]'

Two fields do the heavy lifting. A device with "source": "Linked" was meshed in from another hub, and its remoteDeviceUrl names that hub and the id it holds there — that pair is the whole mesh map. A device with "hubMesh": true is shared outward, which is the opposite relationship and easy to misread.

Notes on doing this safely:

– Everything above is a GET. Keep it that way. In particular do not open http://10.2.3.51/installedapp/configure/520/mainPage just to look: the app inventory is available from hub2/appsList, and that page carries a "Create New Access Token" button that would invalidate the token in our config and stop all collection.

– hub2/hubData returns the hub's dashboard access token and the registered account email. Do not paste raw responses into commits, issues, or documents.

– python -m app.hubitat --list-devices will not answer any of these questions. Despite the name it filters to devices reporting a temperature, so it cannot see a switch, outlet, or fan at all.